1. Singularity Waveforms & Energy Proofs

- Waveform Sketching (ut & rt):

  - Step ut-t0: Instantaneous vertical jump of +1 at t=t0.

  - Ramp rt-t0: Slope increases by +1 starting at t=t0.

  - Strategy: Construct time intervals bounded by critical points (where steps/ramps trigger). Track the cumulative value and slope across each interval.

- Energy Signal Verification:

  - Condition: A signal xt is an energy signal if its total energy satisfies 0<E<∞.

  - Method: Split integration across piece-wise intervals determined in the sketch:

E=-∞∞|xt|​2 dt=ititi+1|xit|​2 dt

  - Result: Evaluate definite integrals; if E yields a finite positive number, xt is an energy signal.

2. Double-Sided Amplitude & Phase Spectra

- Conversion to Canonical Cosine Form:

  - Expand product terms using product-to-sum identities:

sinAcosB=12sinA+B+sinA-B

  - Convert all sine terms to cosine using sinθ=cosθ-π2.

  - Absorb negative leading coefficients into phase using -cosθ=cosθ±π.

- Euler Expansion to Exponential Form:

  - Substitute each term Acosωt+θ=A2ejωt+θ+A2e-jωt+θ.

- Plotting Spectra (f-axis in Hz):

  - Frequencies: Calculate fk=ωk2π for positive and negative pairs (±fk).

  - Amplitude Spectrum: Plot vertical stem of height |ck|=Ak2 at f=±fk.

  - Phase Spectrum: Plot vertical stem of angle θk at f=+fk and -θk at f=-fk.

3. Continuous LTI System Responses via Convolution

- Convolution Integral:

yt=xt*ht=-∞∞xτht-τ dτ

- Step Response st [where xt=ut]:

  - Set integral bounds over active step duration (0≤τ≤t for t≥0):

st=0thτ dτut

- Arbitrary Causal Input Response yt:

  - Use uτ and ut-τ to establish non-zero integration bounds (typically 0,t).

  - Expand integrand algebraically before integrating.

  - Evaluate limits at τ=t and τ=0, then append ut to enforce causality.

4. System Analysis via Linearity & Time-Invariance (LTI)

- Core Principles:

  - Time-Invariance: If xt→yt, then xt-t0→yt-t0.

  - Linearity: If xt=a⋅x1t+b⋅x2t, then youtt=a⋅y1t+b⋅y2t.

- Step-by-Step Method:

  - Deconstruct Input: Express non-standard input (like rectangular pulse Π) as a sum of shifted unit steps:

Πt-t0W=ut-t0-W2-ut-t0+W2

  - Shift Known Response: Replace t in the given step response yt with t-t1 and t-t2.

  - Superimpose: Subtract the delayed responses to obtain total system output.

5. Fourier Series, Periodicity, & Parseval's Power Theorem

- Fundamental Period (T0) & Harmonically Related Terms:

  - Individual periods Ti=2π|ωi|.

  - Fundamental period T0=LCMT1,T2,…. Fundamental frequency f0=1T0.

- Extracting Fourier Coefficients (ck):

  - Match terms directly to complex exponential Fourier series xt=ckejkω0t.

- Parseval's Power Theorem:

  - Average Power P equals the sum of squared magnitudes of all Fourier coefficients:

P=1T00T0|xt|​2 dt=k=-∞∞|ck|​2

- Invariance Rule:

  - Modifying phase angles (θk) or rearranging component frequencies does not change total average power P, because |ck|​2 remains unaffected.