Course Project Report: Fall & Call - Multimodal Emergency Detection

1. Aim

The goal of this project is to develop a two-stage, power-efficient emergency detection system for the

Arduino Nano 33 BLE Sense. The system aims to detect a physical fall using an IMU (Accelerometer and

Gyroscope) and subsequently verify the distress signal using Voice Keyword Spotting (KWS). By using a

"Sensory Gating" approach, the device preserves battery life and privacy by only activating the microphone

after a high-G impact and a significant orientation change (tilt) are detected.

2. Data Preparation and Processing

2.1 Dataset used

* IMU Data: A custom dataset was collected using IMU_Fall_Capture.ino. It consists of 120-sample windows

(~1 second) capturing the descent, impact, and post-fall stillness. Classes include "Fall" and "Normal"

(sitting, jumping, flat drops).

* Audio Data: A custom voice dataset was collected using Audio_Keyword_Capture.ino. It includes four

classes: "HELP", "EMERGENCY", "CANCEL", and "BACKGROUND" (noise/silence).

2.2 Selecting and setting the parameters

The following parameters were utilized for the two models:

Stage 1: IMU Classifier

* Sampling Rate: ~119 Hz

* Window Size: 120 samples (1 second)

* Pre-Trigger Buffer: 40 samples (0.33s) to capture the descent phase.

* Features: 6 (ax, ay, az, gx, gy, gz)

Stage 2: Audio Classifier (MFCC)

* Sampling Rate: 16,000 Hz

* N_MFCC: 13

* Frame Size: 512 (32ms)

* Hop Length: 256 (16ms / 50% overlap)

* N_MELS: 26

2.3 Extracting data to be used

Data was collected using a 65/35 split strategy. 65% of samples were recorded in quiet environments to

establish a "clean" baseline, while 35% were recorded with common household background noises (TV, fans) to

ensure robustness.

3. Model Training

3.1 Model Architecture

* IMU Model: A Multi-Layer Perceptron (MLP) with a Dense input layer, a Dropout layer (0.2), a hidden Dense

layer (16 neurons), and a Softmax output. This was chosen for its minimal memory footprint.

* Audio Model: A 2D Convolutional Neural Network (CNN) similar to the Lab 4 architecture: Conv2D ->

MaxPooling -> DepthwiseConv2D -> Flatten -> Dense.

3.2 Training results

[INSERT TRAINING ACCURACY/LOSS GRAPHS HERE]

* IMU Model Accuracy: [E.G. 94.2%]

* Audio Model Accuracy: [E.G. 89.5%]

3.3 Evaluating the model

[INSERT CONFUSION MATRICES HERE]

* The IMU model was specifically tested against "Flat Drops" to ensure the orientation logic successfully

rejected non-fall impacts.

* The Audio model was tested against "TV Chatter" in the Background class to minimize false triggers.

4. TensorFlow Lite Model Conversion

Both models were converted to .tflite format using the TFLiteConverter.

* Quantization: [E.G. Weight Quantization / int8]

* Final Model Sizes:

* fall_model.tflite: [E.G. 4 KB]

* emergency_model.tflite: [E.G. 18 KB]

5. Arduino Implementation

5.1 System State Machine

The system operates using four primary states:

1. IDLE: Monitoring Accelerometer for IMPACT_THRESHOLD > 4.5G.

2. VERIFYING: Waiting 1.5 seconds after impact to calculate the Tilt Angle and verify Stillness.

3. LISTENING: If verified, Stage 2 activates the PDM microphone for keyword recognition.

4. ALARM: Triggers an external LED/Actuator if "HELP" or "EMERGENCY" is recognized.

5.2 Orientation Verification Logic

To calculate the 90-degree tilt, a Dot Product calculation is performed between the initial standing vector

(captured during calibration) and the current gravity vector after the fall.

1 float dot = (ax * standX) + (ay * standY) + (az * standZ);

2 float angle = acos(dot) * 180.0 / PI;

5.3 Overall Flow

The loop() function remains in a low-power state polling the IMU. Only when the mathematical "Fall

Signature" is confirmed does the computationally expensive PDM microphone and CNN inference wake up.

6. Conclusion

[SUMMARIZE THE SUCCESS OF THE PROJECT. DID THE 2-STAGE GATING REDUCE FALSE ALARMS COMPARED TO A SINGLE-STAGE

TRIGGER?]