SC2107 Lab4 Assignment Sheet (to be submitted to NTULearn before next lab)

Name:	Jonathan Low Seng Kian		      Lab Group:	TCMA       Date: 24/10/2025

- Section 6.1. What is the issue when an obstacle is placed too close to the IR sensor?  What can you do to prevent such ambiguity?

When an object is too close to the sensor, the output voltage decreases as the object gets close as seen in the graph in the briefing slides, the sensor has non-monotonic behaviour. This could lead to 2 different interpretations of the same voltage. To prevent this, you can either set the IR sensor to be behind more, or add some physical bumper that prevents objects from getting too close.

- Section 6.1. What is the purpose of the 10uF decoupling capacitor?

Its to stabilize the power supply to the IR sensor. It acts as a low pass filter to filter out high-frequency noise and smooth out voltage fluctuations. This is to ensure the ADC readings are accurate and stable.

- Section 6.2.  Which port pins is ADC Ch12, 16 and 17 inputs mapped to? Which PxSELx and what settings are required to configure the pins to ADC function?

ADC ch12 is P4.1, ch16 is to P9.1 and ch17 is P9.0. To enable ADC function, PxSEL0 and PxSEL1 registers need to be set to 1.

- Section 6.2. With respect to the ADC on MSP432, what are the two stages involved in every Analog to Digital Conversion of an Analog signal?

The two main stages for the Successive Approximation Register (SAR) ADC in the MSP432 are:

1. Sampling: The "sample-and-hold" circuit captures the instantaneous analog voltage from the input pin.

2. Conversion: The captured voltage is converted into a digital value by successively comparing it against a series of internally generated reference voltages.

- Section 6.3. What does the function LPF_Calc() does? What are the initial values of the buffer associated with LPF_Calc()?  Why do we need this function?

LPF_Calc() implements a digital Low-Pass Filter. It smooths the raw ADC readings by averaging them over time.

Initial Values: The filter is initialized by LPF_Init(raw_data, s). The buffer is initialized with the first raw ADC reading to ensure the filter starts at the current sensor value, preventing a sudden jump in output.

Purpose: The IR sensor's output can be noisy. The LPF removes this noise and provides a much more stable and reliable reading, which is essential for accurate distance calculations.

- Section 6.3.  Describe the calibration process and algorithm you used to estimate the actual distance based on the IR Sensor value.

Process: The calibration involves fitting a hyperbolic model to the sensor's non-linear response. This is done by:

1. Recording a set of ADC values (the x values) at several known, measured distances (the y values).

2. Using a curve-fitting algorithm to find the coefficients A and B for the hyperbolic equation y = A / (x + B).

Algorithm: After calibration, the distance y can be calculated from any new ADC reading by formula. The LeftConvert, CenterConvert, and RightConvert functions in the C code use this algorithm to convert the filtered ADC values into an estimated distance in millimeters.

- Section 7.2.  Which timer capture input (Timer and Channel number) does P10.4 and P10.5 correspond to?

P10.4 corresponds to Timer_A3, Capture Channel 0 (TA3.CCI0A).

P10.5 corresponds to Timer_A3, Capture Channel 1 (TA3. CCI1A).

- Section 7.2. Which edge (falling, rising, both) is the timer input capture configured to trigger on?  What happens when a capture event occurs?

The capture is configured to trigger on the rising edge of the input. When a rising edge is detected:

1. The current value of the Timer_A3 counter is automatically copied into the corresponding capture register (TA3CCR0).

2. An interrupt is triggered, causing the PeriodMeasure0 ISR to run.

3. The ISR calculates the time difference between the current and previous capture to determine the period of the signal.

- Section 7.2. Why is the calculated value of pulse duration derived from the timer capture values, not a constant value i.e. kept changing?

The pulse duration (period) represents the motor's speed. This value constantly changes because the motor's speed is never perfectly stable, even when given a constant power level. The fluctuations are caused by minor physical imperfections in the motor and gears, variations in friction, and slight changes in the electrical load or surface the robot is on.